Plumbing Walk-Thru Checklist

- Soft water

  - Discuss and verify with Owner which fixtures have soft (or hard) water run to them

- Hot water

  - Discuss recirculating system and Owner’s expectations for how quickly hot water is available

- Hose Bibs

  - Discuss quantity and location of exterior hose bibs

  - Verify garage interior is to have a hose bib (hot/cold?)

- Sprinkler System

  - Verify location and size of backflow preventer and sprinkler T

- Interior Drains

  - Confirm locations of PVC drains and make sure they are in non-public areas so sound of running water is not noticeable

- Shower Heads

  - Confirm height and location of shower heads and valves

- Master Shower

  - Confirm location of shower head(s)

  - Confirm location of valves and volume controls

  - Confirm location of body sprays as applicable

  - Confirm location of master tub and tub filler

  - Verify steamer location (unit, controls, overflow drain) as applicable

- Garage Trench Drains

  - Confirm location and size

- Swimming Pool

  - Confirm requirements for pool subcontractor – gas, water, sanitary

- Laundry Room

  - Verify wash box location

- Floor Drains

  - Verify and double check all locations

  - Verify appliances (ice machines, etc.)

- Sitework

  - Yard hydrants

    - Discuss with Owner and confirm location of any potential yard hydrant (on large lots)

  - Water features

    - Discuss procedures and codes relevant to water features

- Gas Lines

  - Confirm size of gas lines and locations

  - Discuss any future locations with Owner (gas grill, heaters, etc.)

  - Fireplaces, heaters, cooktop (future if electric)

- Water heaters

  - Discuss and verify type with Owner – type and location of venting

- Water lines

  - Verify options (shut-offs) on any lines that are at risk of freezing (garage faucets, etc.)

- Fixtures

  - Confirm Owner’s final approval before ordering fixtures

  - Discuss and fixtures that require special hook-ups (power operated stools, etc.)

- Reverse Osmosis Water Treatment

  - Discuss locations of use (drinking water, appliances) and what is hooked up to it