Tips for Reviewing Precast Shops – What to Look For

- Note: You can always cut pieces. You can’t grow pieces.

- Are the joints centered on the windows? You want them to be.

- On front elevations, be picky about where corner joints are (you don’t want to see them); question if there are things you would not want to see everyday

- Confirm areas of overlapping profiles

  - Profile thicknesses thick enough to accept other profiles

- Confirm bed depth works with profiles depths

  - Be aware of full depth vs thin stone

- Confirm fireplace pieces against the approved fireplace model – you want these dimensions to be very precise